## Potential NSDI/Government Value-Added Products

| DATA LAYER | WVSAMB DELIVERABLES (boldfaced text) GOVERNMENT VALUE- ADDED PRODUCTS (numbered text) (All SAMB data projected to WV State Plane Coordinate System) | CATALOG UNITS (Seamless) |
| --- | --- | --- |
| # OrthoImagery | 400’ design scale orthophotos, 2-foot GSD, true color, GeoTIFF/ MrSID (1) Reprojected to UTM Zone 17, Reindexed to USGS quarter quad index | 10,000’ x 10,000’ Tiles 3.75-Minute Quarter Quad |
| Elevation | Mass points and breaklines (pre-TIN format; DGN files) (1) DEM ASCII File Format (mass points and breaklines) (2) ESRI TIN format (3) 5-meter DEMs. Incorporate into National Elevation Dataset. Hydro Enforced. (4) Hillshaded Terrain (5) Contours derived from elevation data (6) Ridgeline breaklines for boundary datasets | 10,000’ x 10,000’ Tiles 10,000’ x 10,000’ Tiles 3.75-Minute Quarter Quad 1/2 deg. X 1/2 deg. Blocks 1/2 deg. X 1/2 deg. Blocks 3.75-Minute Quarter Quad County or State |
| # Transportation | Addressable road centerlines, RRs, bridges, overpasses. (1) Statewide Road Centerline Database (2) ESN Zones (Fire, EMS, Police) (3) Locator-based datasets (Businesses, churches, etc.) (4) Census TIGER/Line Transportation files | 50,000’ x 50,000’ Tiles County and State Emergency Zones State Census Admin. Boundaries |
| Hydrography | Visible water bodies such as rivers, stream, lakes, etC. (1) National Hydrography Dataset (NHD) | 50,000’ x 50,000’ Tiles Watersheds |
| Structures | Centroids and polygons for large structures. Addressing component. (1) Incorporated into cadastral and Census databases (2) Geographic places (3) Flood At-Risk Structures | 50,000’ x 50,000’ Tiles County State State |

Geographic Extent:  Initial data distribution of SAMB project data will be in nested tiles.  As the datasets mature with added attribution and functionality, the appropriate data stewards probably will manage the seamless vector data by (1) political boundaries, (2) watershed boundaries, (3) emergency service zones, (4) Census administrative boundaries, or the popular (5) USGS index referencing system.  Raster digital elevation data usually is managed as latitude/longitude grid blocks.

Seamless Data Distribution:  Internet mapping applications permit seamless data extraction.  A Seamless Data Distribution System provides custom-generated digital products based on user specified geographic extents and user specified datasets.  Seamless digital data can be disseminated to users in several optional formats for delivery via web downloads.

Commercial Value-Added Products:  In addition to government value-added products, numerous vendors will want to develop value-added products from the SAMB project data:  GDT databases used for MapQuest, emergency management applications like E-Team, enterprise transportation applications, etc.

WVGISTC   4-16-03